ADOT Guidance

This template is designed for construction projects seeking coverage under the ADEQ or EPA Construction General Permits for one of the following reasons:

1). The construction site has one or more outfall within ¼ of a mile upstream of a protected surface water that is a WOTUS and is listed as impaired or not-attaining or an Outstanding Arizona Water (OAW), or as otherwise specified of an impaired or Outstanding Arizona Water as defined by ADEQ;

2). An operator discharging to a non-WOTUS protected surface water has chosen to conduct routine analytical monitoring to demonstrate that discharges do not exceed Surface Water Quality Standards. Permittees discharging to non-WOTUS protected surface waters are subject to state requirements only, per A.R.S. 49-255.04(C), enforceable solely by ADEQ; or

3). For sites discharging dewatering water to “sensitive waters” (i.e., receiving waters listed as impaired for sediment or a sediment-related parameter as defined by the EPA, receiving waters designated as a Tier 2, Tier 2.5, or Tier 3 for antidegradation purposes, or receiving waters located within Lands of Exclusive Federal Jurisdiction).

Retain this plan in the Project Stormwater Pollution Prevention Plan (SWPPP). A place holder has been provided in Attachment 13 of the SWPPP.

Monitoring
Sampling and Analysis Plan

As Required Under

[Select applicable Construction General Permit type]

For
[Insert project name.]

| Federal Aid No.: |  |  | [Insert no.] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ADOT Project No.: |  |  | [Insert no.] |  |  |  |
| ADOT District: |  | [Insert District Location] |  |  |  |  |  |
| Route: | [Insert no.] | Milepost (Start): |  | [Insert no.] |  |  | Milepost (End): |  | [Insert no.] |  |  |

Submittal Date:
[Select submittal date.]

Identify Receiving Water/Jurisdiction:
[Select applicable text.]

Prepared by:
[Insert name.]
[Insert company/organization.]
[Insert street address.]
[Insert city, state zip code.]

Arizona Department of Transportation
205 S. 17th Avenue
Phoenix, AZ 85007-3213

Contents

ADOT Guidance

After you have compiled your entire SAP, please follow these steps to update the linked table of contents below:

1.	Place your cursor anywhere in the table of contents.

2.	Press F9, which opens the “Update Table of Contents” dialog box.

3.	Select “Update entire table” in the dialog box and then Click OK.

Introduction	1

Water Quality Parameters	1

Pollutant of Concern	1

Personnel	2

Monitoring Team	2

Training	2

Control Measure Implementation	4

Required Sampling	5

Procedures	5

Calibration	6

Documentation	7

Chain of Custody	8

Monitoring Points	9

Discharge Monitoring Reports	9

References	10

List of Attachments

ADOT Guidance

After you have compiled your entire SAP, please follow these steps to update the linked attachments below:

1.	Place your cursor anywhere in the list of attachments.

2.	Press F9 to refresh.

Attachment A: Phone Tree

Attachment B: Monitoring Plan Map

Attachment C: Discharge Monitoring Reports (DMR)

Attachment D: Chain-of-Custody Records

Attachment E: Calibration and Maintenance Records

Attachment F: Sampling Log

Attachment G: Personnel Training and Qualifications Log

# Introduction

[Select reason for monitoring and sampling plan.]

ADOT has determined that [Insert project name.] consists of construction activities with the potential to discharge stormwater runoff or dewatering water from the project site to a waterbody listed as being [Select applicable water-listing type.]. The operator of this project is required to monitor stormwater runoff from the project and all control measures (structural and non-structural) to ensure these measures are operating effectively and protecting the waterbody. This Monitoring Sampling and Analysis Plan (MSAP) will be provided to the ADOT Engineer for approval and certification prior to the operator seeking coverage under the [Select applicable Construction General Permit type]. This MSAP will be maintained as part of the approved Stormwater Pollution Prevention Plan.

# Water Quality Parameters

[Select applicable text based on the reason for monitoring and sampling.]

## Pollutant of Concern

If you are monitoring for several waterbodies, copy and paste [Select applicable text regarding pollutants of concern for the waterbody being monitored.]

[Select applicable text regarding pollutants of concern for the waterbody being monitored.]

ADOT Guidance

An operator may propose that monitoring is not necessary by providing justification in the SWPPP or SAP. Justification shall include the reason(s) that construction activities will not be an additional source of the pollutant for which the water is impaired. As part of this demonstration, the operator must consider all on-site activities and pollutant sources, as well as any known pollutants (metals, nutrients, etc.) to be present in the on-site soils that will be disturbed. ADEQ must agree with this demonstration and make a determination in writing to the operator. Any ADEQ determinations must be kept with the SWPPP.

ADOT Example Text—No Monitoring

DDT metabolites, toxaphene, and chlordane do not stay in an aqueous state and bioaccumulate rapidly up the food chain. Additionally, most lab reporting limits are not low enough to use results for assessment; therefore, lack of exceedances in the water column does not provide sufficient information about pesticide problems in the stream.

Furthermore, this project is a construction-based activity consisting of excavation and trenching, and is in no way related to agricultural practices.

However, under the AZCGP, construction activities within ¼ of a mile of water bodies and their tributaries classified by ADEQ as either an Outstanding Arizona Water (OAW) or an Impaired Water require monitoring to determine if construction site control measures are effective in protecting water quality of the nearby surface receiving water. Monitoring of any discharges for this site will include the standard qualitative parameters of color, clarity, odor, oily sheen, solids and foam as required by the permit to be performed with an inspection. Each of these parameters are qualitative observations recorded on a separate Inspection Report or in a Discharge Monitoring Report (DMR). All ancillary reports and observations will be submitted to ADEQ upon request.

ADOT Example Text—Monitoring

Because this waterbody has been listed for the impairment of turbidity, the operator will sample for the listed parameter using an industry standard turbidity meter. Because the project will employ control measures specifically designed to reduce sediment from leaving the site, this project does not foresee the construction activity to produce added sediment to the listed waterbody. However, turbidity testing will be performed a minimum of two times per wet season (four times annually) when stormwater exits the site through a discharge in sufficient enough quantities to collect a sample.

This Sampling and Analysis Plan (SAP) has been developed to identify the locations of monitoring points, water quality parameters, listed impairments, sampling procedures, detection limits, and potential sources of the listed pollutant at the construction site.

Impaired Waters require monitoring to determine if construction site control measures are effective in protecting water quality of the nearby surface receiving water. Monitoring of any discharges for this site will only include sampling for turbidity. This parameter is quantitative and will be recorded on a separate Discharge Monitoring Report (DMR) in the industry standard Nephelometric Turbidity Units (NTUs). All other ancillary reports and observations will be submitted to ADEQ upon request.

# Personnel

## Monitoring Team

The duly authorized representative for this project is [Insert Rep’s Name]. The Delegation of Authority Form and documented experience is located in Attachment 10 of the SWPPP.

| Company: | [Insert company name.] |
| --- | --- |
| Responsible Party: | [Insert name of individual who prepared this Sampling and Analysis Plan.] |
| Address: | [Insert address (Street, Suite/Unit/etc., City, State Abbrev. Zip Code).] |
| Phone: | [Insert phone number (111-222-3333).] |
| E-Mail: | [Insert hyperlinked e-mail address.] |
| Responsibilities | Supervise and direct all activities pertaining to [Insert Project Name], monitoring, and implementation including control measure installation and maintenance and correction to comply with water quality standards for conducting sampling, tracking, handling, and report keeping and submittal. |

## Training

Before any monitoring is to take place, monitoring personnel will be trained for proper usage of field monitoring equipment and procedures for visual monitoring. This plan contains a contact list (Attachment A) that is to be used to contact and communicate any non-compliance with the permit or monitoring plan such as exceedances in testing parameters. The phone tree also clearly defines the notification responsibilities of the individuals listed.

Familiarity with this plan, the construction general permit, monitoring requirements, and proper techniques are of the utmost importance for obtaining accurate measurements and information, and to provide the highest level of accuracy in both recording and reporting findings. To maintain consistency in recording and reporting findings, the monitoring should be performed by only one or two individuals. However, anyone associated with monitoring should be appropriately trained in documentation requirements, quality assurance/quality control techniques, proper use of testing supplies and meters, proper technique of filling out a monitoring report, evaluation of the results, and the proper cadence when dealing with chain of custody.

Before scheduled monitoring, all associated equipment should be tested and any necessary maintenance performed to ensure proper working condition. All monitoring equipment shall be kept in a designated place. The sampling equipment used for this project requires proper calibration on each piece of equipment as specified by the EPA and manufacturer standards. Additionally, the turbidity meter will be cleaned and serviced according to the manufacturer’s recommendations. Calibration and Maintenance records should be kept in Attachment E associated with this monitoring plan.

To be considered a qualified person for this project, responsible for conducting inspections or monitoring, the listed Monitoring team member must either:

Have completed the EPA construction inspection course developed for this permit and have passed the exam; or

Hold a current valid construction inspection certification or license from a program that, at a minimum, covers the following:

Principles and practices of erosion and sediment control and pollution prevention practices at construction sites;

Proper installation and maintenance of erosion and sediment controls and pollution prevention practices used at construction sites; and

Performance of inspections, including the proper completion of required reports and documentation, consistent with the site inspection requirements of the applicable CGP(s).

ADOT current certification under the Erosion Control Coordinator (ECC) program meets the above criteria and the current ECC certificate of each Monitoring Team Member is located in the Personnel Training and Qualifications Log (Attachment G) of this MSAP. The ECC has also met one of the following requirements:

Registration in the state of Arizona as a Landscape Architect, with a minimum of 1 year of experience in the fields of erosion control or sediment transport;

Registration in the state of Arizona as a Professional Engineer, with a minimum of 1 year of experience in the fields of erosion control or sediment transport; or

Certification by EnviroCert International, Inc. as a Certified Professional in Erosion and Sediment Control.

### Sampling Frequency

[Select applicable text to indicate whether your construction site is within ¼ mile of an Outstanding Arizona Water (OAW), within ¼ mile of Impaired Water, within ¼ mile of a Not-Attaining Water, or EPA dewatering.]

### Sampling Equipment Training

As a member of the Monitoring Team, I have been trained to operate, and record Monitoring equipment listed here:

| [Select “Yes” or “No.” Do not leave blank.] | Turbidity meter |
| --- | --- |
| [Select “Yes” or “No.” Do not leave blank.] | pH meter |
| [Select “Yes” or “No.” Do not leave blank.] | Clean technique |
| [Select “Yes” or “No.” Do not leave blank.] | Chain of custody |
| [Select “Yes” or “No.” Do not leave blank.] | Preservation |
| [Select “Yes” or “No.” Do not leave blank.] | Calibration procedures |
| [Select “Yes” or “No.” Do not leave blank.] | Field note documentation |
| [Select “Yes” or “No.” Do not leave blank.] | Other [Insert training type in parenthesis] |

I also have the following training credentials or professional certifications demonstrating my abilities to perform Monitoring on this project:

[Select full suite of professional registrations. Delete those that are not applicable. Use the “Other” category to list registrations listed as options here.]

# Control Measure Implementation

As part of this project, the construction activities include required erosion and sediment control concepts identified in the SWPPP in an effort to minimize both on- and off-site flows within or through disturbance areas and to attenuate on-site runoff, thereby allowing sediment to settle and/or be filtered out before leaving the site. Disturbance will be limited to active and identified construction areas, including equipment staging areas. The remainder of the site, including vegetation, will be preserved in its natural state to the greatest extent possible. Upon completion of the project, disturbance areas may be seeded and permanent control measures will remain where appropriate in order to achieve final stabilization. It is anticipated that run-on will be encountered from other adjacent properties during the course of the project. [Select applicable storm-drain text.] are present within and/or adjacent to the planned work areas.

Control measures will combine erosion control and sediment control to maximize retaining of sediment and pollution from leaving the construction site. Pollutants are only those associated with a construction nature and potential pollutants are detailed in the SWPPP document.

# Required Sampling

Benchmark threshold is not an effluent limitation. Rather, it is an indicator that the dewatering controls may not be working to protect water quality, which the operator must investigate and correct as appropriate. A benchmark exceedance is not a permit violation. However, if a benchmark exceedance triggers Corrective Action, failure to conduct any required action is a permit violation.

[Select applicable text for EPA-CGP Benchmark Monitoring.]

At any time prior to or during your coverage under the EPA-CGP, you may request that the EPA approve a benchmark for your site that is higher than 50 NTUs if you have information demonstrating the higher number is the same as your receiving water’s water quality standard for turbidity. Unless the EPA approves an alternate benchmark, you will be required to use the standard 50 NTU benchmark. To request approval of an alternate benchmark, you must submit the information in Appendix K of the EPA-CGP to your applicable EPA Regional Office (Region 9).

Include the current turbidity water quality standard that applies to your receiving water and the source/citation. For instance, if your site is discharging to a water for which the water quality standard is that turbidity may not increase above ambient levels by more than 20 percent, you would reference “Water Quality Standards for the Made up River, Chapter XYZ, Section 1234.56.”

[Select applicable text for required sampling.]

## Procedures

ADOT Guidance

Because each parameter has different techniques and handling procedures, include the proper collection and handling procedures identified to be used with that parameter.

You must include written procedures for each of the four items below

1.	Sample collection (equipment and containers, calibration procedures, document site conditions during sampling, fieldnotes and conditions under which the sample was taken)

2.	Preservation (sample preparation to meet holding times)

3.	Tracking (including chain-of-custody procedures)

4.	Handling (packing, transporting and shipping procedures to maximize sample integrity)

ADOT Example Text

Measuring turbidity only evaluates the optical property of water and the amount of light that is scattered and absorbed by particles in a sample. This project has chosen to use a turbidimeter to measure the average turbidity in NTUs (nephelometric turbidity units) with the turbidity meter set on signal averaging and to record the reading in NTUs. Turbidity measurement does not require any sample preparation other than using a sealed glass container provided by the manufacturer to fit in the turbidity meter. The sample bottles will be shaken thoroughly before each analysis. Glassware will be cleaned with appropriate materials to avoid scratches and dust accumulation. No use of chemicals or other materials will be used to clean the bottles unless instructed by the manufacturer.

Sample collection requires a sample to be grabbed in an undisturbed area prior to entering the listed water body. In the case where a discharge enters the water body prior to sample collection, the sample will be taken using plastic gloves and goggles for personal safety and in an area that has not been disturbed (overtly turbid). The sample is simply poured into a glass tube, placed inside the turbidity meter, and the result is read directly from the instrument.

If the turbidity samples are required to be sent to an accredited lab for reading, the samples will be held in the dark on ice or at 4° C. In this condition, the turbidity samples are good for up to 2 days.

Upon completion of all monitoring for this event, a Discharge Monitoring Report will be populated and any detailed notes will be made in the Sampling Log.

Sample bottles will be handled with care to avoid contamination or damage. Marks and scratches on sample bottles will change the optical characteristics of the glass or cause inaccurate results.

Silicone oil is required for this application and is provided by the manufacturer to have the same refractive characteristics as glass. Excess oil will be removed with a lint-free cloth. The result should be a sample cell surface with all imperfections filled and always handled at the top of the bottle by the cap.

Included in this plan are the written procedures for sample collection. When samples must be taken, we will use [Insert applicable equipment type (e.g., turbidity meters, pH meters, auto samplers).]. Samples will be collected in [Select applicable container type for collection.] containers that have been approved for the collection and preservation of the impairment.

## Calibration

ADOT Guidance

For each piece of equipment that requires calibration, identify the calibration procedures and reference the Calibration Log attached to this SAP.

ADOT Example Text

Calibration of the turbidity meter is required for accurate turbidity measurement, and instrument calibration should be verified on a monthly basis. If calibration indicates significant deviation from the standard (greater than ±10%), the meter will be thoroughly cleaned and recalibrated. If calibration errors persist, the manufacturer will be contacted. At a minimum, regardless of calibration results, turbidimeters should be thoroughly cleaned and calibrated at least quarterly.

QA/QC Considerations:

Randomly select 5 to 10 percent of the samples and collect lab replicates for them. Lab QA/QC will involve selecting duplicate analysis and the calibration of all equipment used with a reference solution.

All monitoring instruments and equipment (including the operator’s own field instruments for measuring pH and turbidity) will be calibrated and maintained in accordance with the manufacturer's recommendations.

The monitoring equipment used for this project requires proper calibration on [Select applicable duration of calibration.] as specified by EPA standards. Additionally, the monitoring equipment will be cleaned and serviced according to the manufacturer’s recommendations.

### Monitoring Equipment Calibration

The manufacturer’s operating instructions will be followed for equipment service and calibration. The monitoring equipment for this project that requires calibration includes [Select applicable equipment needing calibration.].

[Select applicable text regarding type of calibration.]

[Discuss protocols for keeping the monitoring equipment clean and calibration techniques for each piece of equipment.]

ADOT Example Text

The monitoring equipment, including sample tubes to be used with the available instrument, must be of clear, colorless optical glass. They should be kept scrupulously clean, both inside and out, and be discarded when they become scratched or etched. They must not be handled at all where the light strikes them, but should be provided with sufficient extra length, or with a protective case, so that they may be handled. Tubes should be checked, indexed, and read at the orientation that produces the lowest background blank value.

[Insert the manufacturer’s calibration process and steps for proper calibration of each piece of monitoring equipment you will be using.]

ADOT Example Text

## Documentation

ADOT Guidance

Prior to performing any sample collection, coordinate with an accredited laboratory to ensure the lab is licensed to analyze the pollutant of concern. A few other important items to verify are:

1.	Does the lab have the capacity to accommodate your samples within the timeframe you need the data?

2.	Does the lab provide the proper materials (bottles, jars, lids, standardized sample collection directions)?

3.	Does the lab provide an ice chest, if required, and chain-of-custody forms?

4.	What are the business hours of the lab, and do they allow/have the ability to allow drop-offs?

5.	Include sample preservation and protection during transport

The operator will document the site conditions using the Sampling Log in Attachment F of this MSAP. Documentation of the site conditions during sampling will include weather, temperature, conditions requiring the collection of samples, and control measures that have the potential to influence discharges at the site. These documentation notes will identify the conditions during sampling.

For all samples taken, the operator will follow these steps to ensure proper preservation techniques (sample preparation to meet holding times) and handling techniques (packing, transporting, and shipping procedures to maximize sample integrity) for all sampling events.

## Chain of Custody

ADOT Guidance

The chain-of-custody form in Attachment D of this SAP or other forms from an accredited lab may be used, provided that the same information is included.

The operator must be able to produce documentation that verifies possession of samples that are collected, transported, submitted to a lab, etc., that are traceable from the time the samples are collected until they are tested and/or readings are recorded. To maintain and document sample possession, we will follow chain-of-custody procedures. A sample is under custody if:

It is in your possession; or

It is in your view, after being in your possession; or

It was in your possession and then you then locked it up to prevent tampering; or

It is in a designated secure area.

Because we are collecting samples for documentation of CGP compliance, the operator will only collect the number of samples which provide a good representation of the stormwater being sampled. The operator has assigned personnel to accomplish stormwater sampling and the list of these personnel and/or laboratory(s) are included in the Personnel section above that identifies the Monitoring Team. The Monitoring Team will be the only personnel involved in the collection, preservation, transportation, and testing, and data documentation of any samples collected for this project (as few people as possible should handle samples). The field sampler is personally responsible for the care and custody of the samples collected until they are transferred or dispatched properly.

All samples will be accompanied by a chain-of-custody record (Attachment D). When transferring the possession of samples, the individuals relinquishing and receiving will sign, date, and note the time on the record. The chain-of-custody record documents sample custody transfer from the sampler, often through another person, to the analyst at the laboratory. The samples are typically transferred to the sample receiving custodian at the laboratory.

All laboratory analyses shall be conducted according to test procedures specified in 40 CFR Part 136, unless other test procedures have been specified in this general permit.

# Monitoring Points

The operator will conduct analytical monitoring a minimum of two times per wet season throughout the duration of permit coverage. Analytical monitoring is only required when stormwater or snowmelt exits the construction site by way of a discharge point in sufficient quantity to allow for sample collection and analysis.

[Select applicable sampling text for monitoring points.]

ADOT Guidance

All operators are required to sample at least one of their discharge points.

Operators of very large construction sites with 5 or more discharge locations are not required to sample every known point in the affected area.

For sites with 20 or more discharge locations, only 10% (rounded to the nearest whole number) of the outfalls and discharge locations are required for sampling.

Operators of sites with 4 or fewer discharge points are required to sample only one, and sites with 5 to 19 sites require only two samples. These samples must be collected from points that are representative of stormwater discharges from the site.

# Discharge Monitoring Reports

ADOT Guidance

Any portion of the Discharge Monitoring Report (DMR) without data will be considered a violation of the monitoring and reporting requirements. To avoid accruing these violations, all portions of the report must be completed.

Monitoring data must be submitted on an electronic Discharge Monitoring Report (eDMR) via a myDEQ account as prescribed in the 2020 CGP. Copies of the analytical test results will be maintained with the SWPPP records and with the Monitoring Plan in Attachment C. Copies of chain-of-custody records will also be kept with this Monitoring Plan in Attachment D and will be inspected to assure all parameters have been analyzed.

All operators subject to analytical monitoring shall submit the results on the electronic Discharge Monitoring Report (DMR) in myDEQ. The DMR shall be submitted within 30 days after receiving laboratory results. In the event no samples are collected during a wet season, the DMR indicating “no data” using the appropriate No Discharge Information (NODI) code(s) shall be submitted no later than:

Winter Wet Season: June 30

Summer Wet Season: November 30

or at the time the conditions of the permit have been met and a Notice of Termination (NOT) is submitted in myDEQ, whichever is sooner. Should there be no discharge associated due to the absence of a discharge in sufficient amounts to obtain a sample, a DMR will indicate this and must be submitted to ADEQ regardless. All reports must be signed by the responsible party.

ADOT Guidance

The responsible party certifying the DMR must be certified by an eligible person and meet the requirements specified in Appendix B, Section 9, of the Construction General Permit. This can be the ECC, with Delegation of Authority to sign DMRs, or the NOI Signatory. The person making the certification may or may not be the same as the contact person performing the inspection/sampling.

# References

Hach Company. 1999. Portable Turbidimeter Model 2100P: Instrument and Procedure Manual. Revised edition. Cat No. 46500-88. http://www.water-research.net/Waterlibrary/watermanual/turbid2100P.pdf.

U.S. Environmental Protection Agency (EPA). Appendix B: Visual Monitoring Form. EPA 2009. https://www3.epa.gov/npdes/pubs/msgp_monitoring_guide.pdf.

Arizona Pollutant Discharge Elimination System General Permit for Stormwater Discharges Associated with Construction Activity to Surface Waters (AZG2020-001) https://www.azdeq.gov/AZPDES/CGP

Phone Tree

| Resident Engineer |  |  |  |
| --- | --- | --- | --- |
| [Insert Name] [Insert Title] [Insert Address] [Insert Phone No.] |  |  |  |
|  | District Environmental Coordinator |  |  |
|  | [Insert Name] [Insert Title] [Insert Address] [Insert Phone No.] |  |  |
|  |  | Erosion Control Coordinator (Monitoring Team) |  |
|  |  | [Insert Name] [Insert Title] [Insert Address] [Insert Phone No.] |  |
|  |  |  | Accredited Laboratory |
|  |  |  | [Insert Name] [Insert Rep. Title] [Insert Address] [Insert Phone No.] |
|  |  |  | Weather Monitoring Service |
|  |  |  | [Insert Name] [Insert Rep. Title] [Insert Address] [Insert Phone No.] |

Monitoring Plan Map

Monitoring Plan Map

Discharge Monitoring Reports (DMR)

Chain-of-Custody Records

Calibration and Maintenance Records

Calibration Log

ADOT Guidance

If the project has more than one inspector, please supply separate contact information for each inspector. Follow these steps to add a row for each inspector’s contact information:

1.	Left-click your mouse on the outside of the fifth cell of the last row. Hold and drag your mouse from right to left to select the entire row.

2.	Hover your mouse over the selected row and then right-click and select “Copy.”

3.	Right-click again and select “Paste Rows.”

4.	Repeat Step 3 (or press Ctrl+Y) as many times as needed to insert your desired number of additional rows.

| Date | Initials | Equipment Type/Model | Maintenance Needed | Meter and Collection Equipment in Compliance |
| --- | --- | --- | --- | --- |
| [Select calibration date.] | [Insert Initials.] | [Insert equipment type/model.] | [Select “Yes” or “No.” Do not leave blank.] | [Select “Yes” or “No.” Do not leave blank.] |

Sampling Log

Sampling Log

| Project Name: | [Insert Project Name.] |  |  |
| --- | --- | --- | --- |
| AZCON/EPA Number: | [Insert AZCON/EPA No.] |  |  |
| Date of Inspection: | [Select Inspection Date.] |  |  |
| Monitoring Point: | [Insert Point No.] |  |  |
| Monitoring Duration: | Start Time: End Time: | [Select applicable calendar day and then manually update start time.] [Select applicable calendar day and then manually update start time.] |  |
| Monitor’s Name: | [Insert Monitor’s Name.] [Insert Monitor’s Name.] [Insert Monitor’s Name.] |  |  |
| Monitoring Reason: | - Stormwater or snowmelt is exiting the construction site by way of a discharge point [Select applicable text.]. - Dewatering water is exiting the construction site by way of a discharge point. The discharge has the following characteristics: |  |  |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Color |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Odor |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Oily sheen |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Clarity |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Foam |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Turbid |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Floating, settled, or suspended solids |
|  | [Select “Yes,” “No,” “Not Applicable.” Do not leave blank.] |  | Other obvious indicators of stormwater pollutants: [If “yes,” then describe here] |
| Weather at time of this monitoring event? | [Select applicable weather condition.] Temperature at Time of Monitoring: [Insert temperature] degrees Fahrenheit |  |  |
| Have any discharges occurred since the last monitoring event? | [Select discharge language] |  |  |

Personnel Training and Qualifications Log

ADOT Guidance

Populate this sheet and attach credentials for each person performing monitoring on this project.

| Company: | [Insert company name.] |
| --- | --- |
| Responsible Party: | [Insert name of individual who prepared this Sampling and Analysis Plan.] |
| Address: | [Insert address (Street, Suite/Unit/etc., City, State Abbrev. Zip Code).] |
| Phone: | [Insert phone number (111-222-3333).] |
| E-Mail: | [Insert hyperlinked e-mail address.] |

### Sampling Equipment Training

As a member of the Monitoring Team, I have been trained to operate and record Monitoring equipment listed here:

| [Select “Yes” or “No.” Do not leave blank.] | Turbidity meter |
| --- | --- |
| [Select “Yes” or “No.” Do not leave blank.] | pH meter |
| [Select “Yes” or “No.” Do not leave blank.] | Clean technique |
| [Select “Yes” or “No.” Do not leave blank.] | Chain of custody |
| [Select “Yes” or “No.” Do not leave blank.] | Preservation |
| [Select “Yes” or “No.” Do not leave blank.] | Calibration procedures |
| [Select “Yes” or “No.” Do not leave blank.] | Field note documentation |
| [Select “Yes” or “No.” Do not leave blank.] | Other ([Insert Type]) |

I also have the following training credentials or professional certifications demonstrating my abilities to perform Monitoring on this project:

[Select full suite of professional registrations. Delete those that are not applicable. Use the “Other” category to list registrations listed as options here.]